## Products affected

| Trucks in Series |  |  |  |  | Bus in Series |  |  |  | Scania Engines |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | G | P | R | T | 4 | K | N | F |  |
| Miscellaneous: |  |  |  |  |  |  |  |  |  |

# Overspeeding in Stoneridge tachograph v 7.1 (and 7.2)

# Background

There have been some incidences with interruption during overspeeding as a result of a probable software problem with the Stoneridge digital tachograph version 7.1.

# Problem description

During some special circumstances, when the overspeeding warnig is active the warning still active after going back to normal speed.

The problem is timing dependent (depending on the value of internal timer when entering legal speed) and therefore not possible to reproduce every time.

If this error occurs, system event on TCO1 will be trigged on CAN, this will continue until kl30 (power supply disconnected) or if truck exceeds legal speed and thereafter go below this limit.

# Service solution

Disconnection of the power supply to the digital tachograph, or the main power supply from the battery.

It is recommended that the tachograph power supply is disconnected at a workshop where a workshop card can be inserted into the tachograph before disconnecting the tachograph power supply. The reason is that a “power interrupt” event will be stored in the digital tachograph memory when the power supply is disconnected. However, if a workshop card was inserted during this power interruption it can be proven that this “power interruption” event was stored during a workshop visit. Check with local authorities whether a new tachograph calibration is required after a “power interruption” event has been stored in the tachograph memory.

The problem is not solved permanently but temporarily and the action may be needed again and can happen the next day, month or never again.

This problem is solved in version Stoneridge digital tachograph version 7.3.